AI Attendance Tracker Via Face Recognition

Minal Barhate, Soham  Kurulkar, Soham Patil, Snehal Dhande, Suchit Tumbada

Department of Engineering, Sciences and Humanities (DESH)

Vishwakarma Institute of Technology, Pune, Maharashtra, India

Abstract —  . This project presents a simple to operate web-based Face Recognition Attendance System designed to automate the process of attendance marking and make it less tedious by  using computer vision and deep learning techniques and algorithms such as Haar Cascade and Vision Transformer . The system employs a Vision Transformer (ViT) model to extract highly accurate and apt facial embeddings from captured images, enabling accurate and robust face recognition and identification. A Haar Cascade classifier is utilized for real time face detection, while cosine similarity is applied to match extracted embeddings with pre-stored known identities. The embeddings are compared with the help of cosine distance to check if they match the faces already present in the database in the backend. The  solution we have presented integrates a Flask based backend with a browser accessible frontend, allowing attendance to be recorded automatically through live webcam capture without manual intervention. Additionally, the system supports on time  enrolment of new users through a separate tab on the webpage which when opened allows users to open the computer webcam and take a photo of the new student to be enrolled, enabling simple yet effective database expansion and embedding generation. Attendance records are securely stored with timestamps under another tab . The design of this project focuses on  modularity, scalability, and ease of deployment, and ease of use, making it exteremly suitable for academic and organizational environments, and most importantly it being easy to operate makes it easily adaptable to . This system significantly reduces human error, prevents proxy attendance, saves time for teaching staff and demonstrates the practical application of transformer-based vision models in real world biometric systems.

Keywords — Attendance tracking, CNN-model, Deep learning, Haar Cascade, Vision Transformer.

# INTRODUCTION

Attendance management is a fundamental requirement in educational institutions and organizational environments. It is of great importance as it demands students of the educational institute to attend the classes regularly and gain the most benefit from sitting in these classes and the dynamic learnings they provide. Attendance tracking traditionally relied on manual methods such as paper registers or identity cards. These conventional approaches are time consuming, error prone, and susceptible to issues such as proxy attendance and data inconsistency. Furthermore, they significantly take up time which could have been used for efficient learning. With the rapid advancement of artificial intelligence and computer vision, biometric based systems have emerged as a reliable alternative for automating identity verification tasks. Among various biometric modalities, facial recognition has gained significant attention due to its non-intrusive nature and ease of integration with existing camera infrastructure.

Recent developments in deep learning have substantially improved the accuracy and robustness of face recognition systems. Traditional convolutional neural network based approaches, while effective, often struggle with variations in pose, lighting, and occlusion. Vision Transformers originally introduced for image classification tasks, have demonstrated superior capability in capturing global contextual information and learning discriminative visual features. This makes them particularly suitable for facial representation learning in unconstrained environments.

This research proposes a Face Recognition Attendance System that leverages a Vision Transformer based embedding model for reliable face identification. The system integrates real-time webcam-based image acquisition, automated face detection, embedding generation, and similarity-based matching to accurately recognize individuals. A web based interface allows seamless interaction for attendance marking and enrollment of new users, while attendance records are stored with precise timestamps and presented through a structured visual report. The proposed solution aims to enhance accuracy, reduce administrative overhead, and provide a scalable and efficient attendance management framework suitable for real-world deployment.

# LITERATURE REVIEW

Attendance Tracking through the means of facial identification have gained immense attention due to their ability to eliminate many redundancies such as prevention of proxy attendances, save teaching staff’s time and provide a seamless easy hassle-free process of marking the  presence of students in the classroom. Early research such as Smart Attendance System Using Face Recognition which was written by Ghalib Al-Muhaidhri and Javeed Hussain demonstrated the ability to replace traditional attendance taking processes bu using computer vision algorithms like Haar cascades and Open-CV. While these systems were working effectively in controlled environments they failed whenever there was small change in the environment which may have less or more light, shadow placements and other such visual aspects.

With the stark usage of deep learning, researchers began using CNNs which stands for Convolutional Neural Networks which helped in enhancing the ability to extract out facial features in a more effective way which in turn increased accuracy and reliability of the attendance system. Shravan Chandaka and Avinash Rao Thandra​ developed a CNN-based face detection system and recognition system that significantly improved the performance under variations in surroundings. Similarly,Khawla​ Alhanaeea,​Mitha Alhammadi​, Nahla Almenhali​, and Maad Shatnawi​ made the use of transfer learning with prior trained CNN models to achieve a higher accuracy rate and also helped improve the overall generalization of the working of the model. These works conveyed the fact that deep learning is a far superior competitor to handcrafted feature-based approaches.

Furthermore, studies have communicated heavy focus on real-time operation and the ability for the system to scale effectively. In the paper “Real-Time Student Face Recognition Attendance System Using AI​”  published by Dharmaraj K. B., Dhanushree C. N., Revanth H. V., Shashank N., Tejaswini P. M.​ Their project works on a system which integrates CNN-based recognition with a graphical user interface and text to speech aid which helps them achieve an accuracy between 92 to 95 percent in variating classroom environments. In the paper “Enhancing Classroom Attendance Systems with Face Recognition​” written by Jaykumar Patela, Savita Gandhia​, Vishal Katheriyaa​,Parth Pataliyaa​ and  Ankan Majumdara​, they mention evaluating deep learning based systems and highlight the challenges related to classroom dynamics such as student movement, occlusions, and changes in lighting or appearance of shadows and what not.

To improve and further enhance face detection and alignment, Ubongn David Essien  and Godwin Okon Ansa ​ in their paper “A deep learning-based face recognition attendance system” mention about how they used Multi-Task Cascaded Convolutional Neural Networks which enabled them to achieve reliable face detection and alignment prior to recognition. In addition, Touzene Abderraouf, Abed Abdeljalil Wassim, and Slimane Larabi​ in their paper “An Embedded Intelligent System for Attendance Monitoring​” explored a more easy deep learning model which deployed embedded systems such as Raspberry Pi, demonstrating that resource-efficient face recognition can be achieved for low power environments. These studies indicate a growing interest in optimizing computation efficiency without sacrificing accuracy of the model itself. More recent work by M. A. Hossain, S. M. S. Islam, and R. Debnath in their paper titled “A Deep Learning–Based Real-Time Face Recognition System for Automated Attendance” presented a deep learning-based real-time face recognition attendance system achieving over ninety seven percent  accuracy using CNN architectures and extensive dataset evaluation. The widespread adoption of frameworks such as TensorFlow, Keras, and OpenCV across these studies reflects the maturity and standardization of deep learning tools for face recognition applications.

As stated above, although existing models give and show high recognition accuracy, most systems primarily focus on backend model performance and lack a easy to use, seamless web-based approach, real time user interaction, and dynamic, yet easy student enrollment. In addition, many approaches rely heavily on CNN-based architectures, which while being effect often struggle with long range feature dependencies and become hard to scale as an entire project which could have been applied in solving real world problems.

In contrast, the proposed FaceTrack AI system integrates Vision Transformer based embeddings, which provide enhanced and easy to use features as compared to CNN models. Furthermore, unlike earlier systems, the solutions we have provided offers a fully working web-based interface, live webcam capture, real-time attendance marking, automatic CSV logging and on the fly easy to perform student enrollment process. thereby addressing key limitations identified in previous studies. This holistic approach makes this unique system as a scalable, user-centric, easy to use, and technologically advanced solution for modern smart attendance systems.

# Methodology/Experimental

## The methodology of the proposed system follows a two-phase approach: student enrollment and attendance recognition.

During the enrollment phase, the system captures a student’s facial image through the webcam. The captured image is preprocessed and passed through a Vision Transformer–based deep learning model to extract a high-dimensional facial embedding. This embedding, along with the student’s identity label, is stored in a persistent database of known faces.

In the attendance recognition phase, live facial images are captured in real time and converted into embeddings using the same deep learning model. These embeddings are compared against the stored embeddings using a distance-based similarity metric. If the computed distance is below a predefined threshold, the face is considered a match and attendance is automatically marked with the current date and time. If no match is found, the system reports the face as unknown and attendance is not recorded.

Below is a simple flowchart of how the attendance tracker works in simple step by step manner

Step 1: Open Web Application

Step 2: Open Webcam

Step 3: Capture Face Image

Step 4: Send face image to Flash server

Step 5: Face detection using Haar Cascade Classifier

Step 6: Face cropping and preprocessing

Step 7: Generate Face Embeddings using Vision Transformer (ViT)

Step 8: Load stored face embeddings from database into embeddings pickle file

Step 9: Compare embedding using cosine distance

Step 10: If face matches then append attendance

into csv file along with timestamp. If face not found show error message on webpage.

# Results and Discussions

A successful model works in which a webpage attendance system is at work. Once you go to the webpage, on the home page you have options to either mark attendance of existing student by simply turning on your webcam and clicking on ‘mark attendance’ or one can enroll a new student and save their face image in backend for future attendance marking. The webpage also includes tabs to view the attendance log which automatically gets appended after every attendance marked along with the timestamp to the right of it. This simple system is of great use and will surely aid in attendance tracking on a day to day basis for teachers and professors.

# Conclusion

The project successfully demonstrates the design and implementation of an automated attendance management system using face recognition techniques. By combining real time face detection with transformer based facial embedding extraction, the system provides a reliable and efficient alternative to conventional attendance marking methods. The use of a Vision Transformer model enables the system to capture rich facial structure, improving recognition accuracy even under moderate variations in lighting and facial orientation.

The proposed web based framework simplifies both attendance marking and user enrollment through an intuitive interface, reducing manual effort and minimizing the possibility of proxy attendance. Experimental observations indicate that the system performs consistently in controlled classroom environments while maintaining acceptable response time and accuracy. Additionally, the modular design allows future enhancements such as database scaling, performance optimization, and deployment in diverse institutional settings.

Overall, this work highlights the practical applicability of modern deep learning models in real-world biometric systems and serves as a foundation for developing more intelligent, secure, and scalable attendance solutions in academic and organizational domains.

X . Acknowledgment

We would like to express our sincere gratitude to the faculty members of the Department of Engineering, Sciences and Humanities (DESH), Vishwakarma Institute of Technology, Pune, for their continuous guidance and encouragement throughout the course of this project. Special thanks to the project guides and mentors for their valuable suggestions, technical insights, and constructive feedback, which greatly contributed to the successful completion of this work.

We also acknowledge the support provided by the institute in terms of infrastructure and learning resources. Lastly, we appreciate our peers and classmates who offered assistance and motivation and also gave insightful feedback during the development and testing phases of the project which has helped immensely in creating a appealing yet efficiently working solution.

References

- Dharmaraj K. B., Dhanushree C., N. Revanth H. V., Shashank N., and Tejaswini P. M., “Real-Time Student Face Recognition Attendance System Using AI,” International Advanced Research Journal in Science, Engineering and Technology (IARJSET), May 2025.

- Patel J., Gandhi S., Katheriya V., Patil P., Ankan, and Majumdar M., “Enhancing Classroom Attendance Systems with Face Recognition,” ScienceDirect (Elsevier), 2025.

- Shravan C., Avinash Rao, and Thandra R., “Face Recognition-Based Attendance System Using Deep Learning,” International Conference on Emerging Computing, 2024.

- Essien U. D. and Ocon A. G., “A Deep Learning-Based Face Recognition Attendance System,” Global Journal of Engineering and Technology Advances, July 2023.

- Abderraouf T., Abdeljalil A., Wassim S., and Larabi S., “An Embedded Intelligent System for Attendance Monitoring,” arXiv Preprint, June 2024.

- Alhanee K., Mitha A., Alhammadi N., Almenhali M., and Shahtawi M., “Face Recognition Smart Attendance System Using Deep Transfer Learning,” Procedia Computer Science (Elsevier), 2021.

- Al-Muhaidhri G. and Hussain J., “Smart Attendance System Using Face Recognition,” International Journal of Engineering Research & Technology (IJERT), December 2019.

- Hossain M. A., Islam S. M. S., and Debnath R., “A Deep Learning-Based Real-Time Face Recognition System for Automated Attendance,” Journal of King Saud University – Computer and Information Sciences (Elsevier), January 2024.